The run-to-failure experiments data were collected at the University of New South Wales in 2019-2020, regarding the development of bearing fault severity assessment methods. The following journal paper [1] contains the description of the experiments and test rig information.

[1] H. Zhang, P. Borghesani, R. B. Randall, and Z. Peng, “A benchmark of measurement approaches to track the natural evolution of spall severity in rolling element bearings,” Mech. Syst. Signal Process., vol. 166, p. 108466, 2022, doi: 10.1016/j.ymssp.2021.108466.

Images of the developing spalls are available in a recently published Master thesis [2].

[2] S. Zhuang, “Tribological investigation on the degradation process of contact fatigue in rolling bearings,” University of New South Wales, 2021. Available at: http://unsworks.unsw.edu.au/fapi/datastream/unsworks:76526/SOURCE02?view=true

Please consider citing these sources if relative to your work.

Data of four experiments, named Test 1 to 4 according to Ref. [1], are stored in four folders respectively. Each folder contains two subfolders, one has all collected signals at 6 Hz, and the other has selected samples at 4 speeds (6,12,15,20 Hz). Each data file is named as vib_aaaaaaaaa_bb.mat:

- aaaaaaaaa: is the running cycle of the shaft in this measurement

- bb: is the shaft speed in Hz

Each data file contains the following variables:

- Fs: the sampling frequency

- accH: acceleration (V) in horizontal direction

- accV: acceleration (V) in vertical direction

- enc1: encoder signal channel 1 (1024 pulses/rev)

- enc2: encoder signal channel 2 (1024 pulses/rev)

- loadCell: the applied radial load on the test bearing

- tacho: tacho signal of the shaft (1/rev)

Sensitivity of each sensor are as follows:

| Sensor type | Sensitivity |
| --- | --- |
| Accelerometer | 10 mV/ms-2 |
| AE sensor | 1 V/mbar |
| Load cell | 2.8 kN/V |

For any further information, please contact the Tribology and Machine Condition Monitoring group of UNSW Sydney.